TASK-3    DATA LOGGER : SENSOR -> SD CARD

CODE : 
 #include <SPI.h>

#include <SD.h>

#include <Wire.h>

#include <RTClib.h>

#include <DHT.h>

#define DHTPIN 4

#define DHTTYPE DHT11

#define LDR_PIN A0

#define SD_CS 10

DHT dht(DHTPIN, DHTTYPE);

RTC_DS3231 rtc;

File dataFile;

void setup() {

Serial.begin(9600);

delay(2000);

dht.begin();

Wire.begin();

if (!rtc.begin()) {

Serial.println("RTC not found");

while (1);

}

if (!SD.begin(SD_CS)) {

Serial.println("SD card failed");

while (1);

}

if (!SD.exists("data.csv")) {

dataFile = SD.open("data.csv", FILE_WRITE);

dataFile.println("Date,Time,Temperature,Humidity,Light");

dataFile.close();

}

Serial.println("System Ready");

}

void loop() {

logDataToSD();

readFileAndPrint();

delay(5000);   // sampling rate

}

void logDataToSD() {

DateTime now = rtc.now();

float temp = dht.readTemperature();

float hum = dht.readHumidity();

int lightVal = analogRead(LDR_PIN);

if (isnan(temp) || isnan(hum)) {

Serial.println("DHT error");

return;

}

dataFile = SD.open("data.csv", FILE_WRITE);

if (dataFile) {

dataFile.print(now.year()); dataFile.print("-");

dataFile.print(now.month()); dataFile.print("-");

dataFile.print(now.day()); dataFile.print(",");

dataFile.print(now.hour()); dataFile.print(":");

dataFile.print(now.minute()); dataFile.print(":");

dataFile.print(now.second()); dataFile.print(",");

dataFile.print(temp); dataFile.print(",");

dataFile.print(hum); dataFile.print(",");

dataFile.println(lightVal);

dataFile.close();

Serial.println("Data logged to SD");

}

}

void readFileAndPrint() {

Serial.println("---- CSV FILE CONTENT ----");

dataFile = SD.open("data.csv");

if (dataFile) {

while (dataFile.available()) {

Serial.write(dataFile.read());

}

dataFile.close();

} else {

Serial.println("File open failed");

}

Serial.println("---- END OF FILE ----");

}

CSV FILE SNIPPET :

## Sampling Rate Explanation

The sampling rate defines how frequently sensor data is collected and stored.

In this project:

- Sensor readings are taken at fixed time intervals

- The interval is controlled in the code using a delay / timing logic

- Each sampling cycle performs:

  - Sensor data acquisition

  - Timestamp retrieval from RTC

  - Writing data as a new row in the CSV file

Choosing an appropriate sampling rate is important because:

- High sampling rate → more data, higher power consumption

- Low sampling rate → reduced power usage and longer system life

For this application, a moderate sampling interval is selected to balance data resolution and power efficiency.

## Power Consumption Considerations

Power efficiency is an important factor in embedded data logging systems.

The following measures help reduce power consumption:

- Sensors are read only during the sampling interval

- SD card is accessed only when writing data

- Continuous SD card access is avoided to reduce current draw

- Lower sampling frequency reduces:

  - Microcontroller active time

  - SD card write cycles

If deployed in battery-powered systems:

- Increasing the sampling interval significantly reduces power usage

- Sleep modes can be added to further optimize energy consumption